# The package install the single-shot cross-term spatiotemporal encoding (xSPENss) sequence, which supports multi-slicing, segment imaging, array imaging, diffusing etc.

# System requirements: Linux 32 or 64 bits (64 bits recommended), Sequence was tested in VNMRJ3.2 and VNMJ4.0. For on-line processing, the sequence requires the matlab MCR v82, which should be downloaded from MATLAB website during the installation with root permit.

# Contact: zhiyong.zhang@weizmann.ac.il  or zhiyongxmu@gmail.com

- Install the package in the terminal, type command “sh /package directory/xSPENssintall.sh”

- Then open VNMRJ, you will see the xSPENss protocol in the label “Spatiotemporal Encoding” as below:

- The layout is almost same as the EPI’s layout.  In the scan options, “Spin Echo” and “Navigator” aren’t working since xSPEN doesn’t need them. The external trigger wasn’t tested.

- Before run experiments, you should run Acquire/Prescan first to adjust the tep for even/odd acquisition, using “xSPENss setup manual” to manually adjust the tep.

- Then play with the scan parameters as EPI scans, such as the following experiments using ex-vivo brain in VNMRJ3.2:

In multi-slicing, the minimum TR requires a gap between slices (no interleave)

In multi-slicing, For TR=5 doesn’t need a gap between slices but with interleaving scan

Segment xSPEN, it is a way to increase the resolution

“zoom-in” xSPEN, no folding effect

- For function xSPEN imaging, please use “Tf” to create the function contrast. Such as  set Tf=0.01, we can get the images as following:

Tf=0.01

- For xSPEN diffusing, please check the diffusion option. Then you can acquire the data, but the processing is not supported yet.